“POX Controller”

Part 1: Create a hub application.

A hub is used for connecting devices in LAN. When a packet arrives at hub, it is sent through all other ports (it means hubs works by flooding the packets) except the incoming port.

Step 1: Create the hub application in “/home/pox/pox/forwarding”: Copy the file “hub-reactive-flowmod.py” in this directory and fill in the missing code as indicated in this file.

Step 2: Run the hub application. Be careful to be in the correct directory.

Step 3: Create a topology of 4 hosts and 1 switch (use “- -controller remote option”).

Step 4: Test connectivity.

Step 5: Check flow table entry at the switch.

Step 6: Ping from h1 to h4 and dump the output from h2, h3 and h4 to prove that flooding is indeed taking place (verify hub behavior). Use the command xterm h1 h2 h3 h4 to open four terminals.

Part 2: Create a switch application.

When we will run our switch application “switch-application-efficient” on top of POX controller, two tables will be maintained. One table will be maintained at the controller and another table will be maintained at the switch. The table maintained at the controller will be “mac to port table” The table maintained at the switch will contain flow entries. Initially both tables will be empty.

Step 1: Create the switch application in “/home/pox/pox/forwarding”: Copy the file “switch-application-efficient.py” in this directory and fill in the missing code as indicated in this file.

Step 2: Run the switch application.

Step 3: Create a topology of 4 hosts and 1 switch (use “- -controller remote option”).

Step 4: Test connectivity (ping to h4 from h1).

Step 5: Check flow table entry at the switch.

Step 6: Verify switch behaviour.

Part 3: Create a firewall application.

We want to block traffic from mac address “00:00:00:00:00:01” to “00:00:00:00:00:03” and from “00:00:00:00:00:02” to “00:00:00:00:00:04” (see “firewall-mac-policies” excel file).

Step 1: Create the firewall application in “/home/pox/pox/forwarding”: Copy the file “firewall-switch-app.py” in this directory and fill in the missing code as indicated in this file.

Step 2: Run the firewall application.

Step 3: Create a topology of 4 hosts and 1 switch (use “- -controller remote option”).

Step 4: Test connectivity (ping to h4 from h1).

Step 5: Check flow table entry at the switch.

Step 6: Verify firewall behaviour using ping commands among all pairs.