Διαχείριση Δικτύων Βασισμένων στο Λογισμικό

10ο εργαστήριο: “Traffic tests”

| ΟΝΟΜΑΤΕΠΩΝΥΜΟ: |  |
| --- | --- |
| Α.Μ.: |  |

Reply 5 out of 6 questions! 2 points each.

Go to https://learning.knetsolutions.in/docs/ryu/#17-traffic-tests---part1 – Section 17: Traffic Tests - Part1.

Copy l4_switch.py file into ryu/ryu/app directory.

Then run the following tests:

## TCP Tests

- Traffic test from h1 to h4

- Bidirectional Traffic test h1 to h4 (sequentially).

- Bidirectional Traffic test h1 to h4 (parallel).

- Traffic test from h1 to h4 with Multiple Sessions.

Provide 4 screenshots with the previous measurement results (not the flow table entries).

## UDP Tests with IPERF

Provide a screenshot with the measurement result.

## Video Stream Traffic Testing

If you are not using “Eirini” VM: Type: sudo apt-get install vlc vlc-bin

Place the video bbb_sunflower in your home/eirini/Downloads directory (or any other directory).

Provide a screenshot like the following, proving that you have managed to stream the video of your choice:

Change to “su – eirini” the respective command and also use the correct directory in the “cvlc command”.

## D-ITG Distributed Internet Traffic Generator

Open https://learning.knetsolutions.in/docs/ryu/#18-traffic-tests-with-ditg - Section 18: Traffic Tests with DITG. Only UDP Part.

Run this test and show the output of the receiver.log file.